VeriGen RTL Benchmark Evaluation

HPC Execution • Multi-Model Scaling • Error Taxonomy • Verification Workflow

Platform: NVIDIA H100 GPU • HPC Cluster (SLURM)

Models Evaluated: VeriGen 2B, 6B, 16B

Modules Tested: 14 Benchmark Modules → 64 Hierarchical Submodules

# 1. Project Overview

This report documents the end-to-end workflow for evaluating VeriGen — a fine-tuned Verilog RTL code-generation model — across three model sizes (2B, 6B, and 16B parameters) on 14 novel RTL benchmark modules. The work was executed entirely on a university HPC cluster equipped with NVIDIA H100 GPUs, managed by the SLURM workload scheduler.

The primary goal was to systematically quantify the quality of automatically generated Verilog RTL code by categorising observed failure modes across a diverse set of hardware design patterns, ranging from simple combinational logic to complex cryptographic cores and pipelined processors.

## 1.1 What is VeriGen?

VeriGen (github.com/shailja-thakur/VGen) is a family of large language models fine-tuned specifically for Verilog RTL code generation. It is built on top of Salesforce CodeGen and fine-tuned on a corpus of open-source Verilog files. Three sizes are publicly available:

- 2B — shailja/fine-tuned-codegen-2B-Verilog

- 6B — shailja/fine-tuned-codegen-6B-Verilog

- 16B — shailja/fine-tuned-codegen-16B-Verilog

All three models share a critical constraint: a hard 2048-token context window covering both the input prompt and the generated output combined.

## 1.2 Motivation and Research Goal

The original VeriGen paper benchmarks the model on relatively simple RTL tasks. This project extends that evaluation to a broader, more difficult set of 14 custom-designed modules that mirror software-level benchmark suites (SPEC CPU2017, MiBench, CoreMark) translated into equivalent hardware designs. The aim is to answer:

- How does output quality degrade as module complexity increases?

- Which failure modes are consistent across all three model sizes?

- Does increasing model size (2B → 6B → 16B) meaningfully improve functional correctness?

- Can hierarchical submodule decomposition mitigate the token limit bottleneck?

# 2. Benchmark Module Selection

The 14 RTL modules were chosen to span a diverse set of computational patterns. The table below maps each module to its hardware design category.

| # | Category | Module | Design Pattern |
| --- | --- | --- | --- |
| 1 | Basic Digital Logic | Parameterized N-bit ALU | Combinational + Registered FSM |
| 2 | Basic Digital Logic | 32-Entry Register File | Memory array, async read |
| 3 | Communication | UART Transmitter (8-N-1) | Baud rate, shift register FSM |
| 4 | Computer Architecture | Multi-Cycle RISC CPU | 5-phase Harvard CPU |
| 5 | Control Logic | Hardwired Control Unit | Combinational decode, casex |
| 6 | Computer Architecture | 5-Stage Pipelined RISC-V | Hazard detect, forwarding |
| 7 | Computer Architecture | Direct-Mapped Cache | Tag/index/offset, write-through |
| 8 | Control Logic | Round-Robin Arbiter | One-hot grant, pointer rotation |
| 9 | Cryptography | AES-128 Encryption Core | S-box LUT, GF(2⁸), 10-round FSM |
| 10 | Cryptography | SHA-256 Hash Core | 64-round compression, K ROM |
| 11 | Error Detection | CRC-32 Engine (IEEE 802.3) | Bit-serial LFSR, reflected poly |
| 12 | Signal Processing | 8-Point Fixed-Point FFT | Cooley-Tukey DIT, Q1.15 |
| 13 | Data Processing | 16×16 Matrix Multiplier | MAC units, pipelined rows |
| 14 | Algorithms | Hardware Bubble Sort | Compare-swap FSM, 8 elements |

## 2.1 Selection Rationale

The modules were selected to cover the principal hardware abstraction layers — from pure combinational circuits (ALU subunits, compare-swap) through registered FSMs (UART, bubble sort) to complex memory hierarchies (cache) and multi-stage pipelines (RISC-V). Cryptographic cores (AES, SHA-256) and signal-processing primitives (FFT, MAC) were specifically included because their correct implementation requires precise constant values and algorithmic structure, making them ideal stress tests for LLM-based RTL generation.

# 3. Token Limit Problem and Submodule Decomposition Strategy

## 3.1 Understanding the 2048-Token Hard Limit

All VeriGen/CodeGen models share an architectural constraint: the total context window — prompt tokens plus generated tokens — cannot exceed 2048 tokens. This is a hard limit baked into the model architecture, not a tunable parameter.

For simple modules, this is workable. A prompt for the ALU might consume ~80 tokens, leaving ~1968 tokens for output. However, for complex modules:

| Module | Prompt Tokens | Output Budget | Result |
| --- | --- | --- | --- |
| N-bit ALU | ~80 | ~1968 | Output fits comfortably |
| UART Transmitter | ~200 | ~1848 | Borderline; output truncated in some runs |
| Multi-Cycle RISC CPU | ~450 | ~1598 | Output cut mid-generation |
| 5-Stage Pipeline RISC-V | ~550 | ~1498 | Only declarations generated; no logic |
| SHA-256 Core | ~480 | ~1568 | Wrong output; K constant mapping broken |
| AES-128 Core | ~420 | ~1628 | Only partial S-box LUT generated |

## 3.2 The Submodule Decomposition Approach

To work around the token limit without changing the model, all 14 benchmark modules were hierarchically decomposed into smaller, self-contained submodules. Each submodule prompt was engineered to consume fewer than 120 tokens, leaving approximately 1920 tokens for the generated Verilog code. This ensured no submodule was truncated mid-output.

The decomposition also mirrors good RTL engineering practice: large digital systems are naturally built from a hierarchy of smaller, independently testable components.

Decomposition result: 14 benchmark modules → 64 hierarchical submodules

### Submodule Hierarchy Table

| # | Module | Submodules | Hierarchy |
| --- | --- | --- | --- |
| M01 | ALU | alu_addsub, alu_logic, alu_shift, alu_flags, alu_top | 5 submodules |
| M02 | Register File | regfile_mem, regfile_top | 2 submodules |
| M03 | UART TX | baud_gen, uart_tx_shift, uart_tx_fsm, uart_tx_top | 4 submodules |
| M04 | Multi-Cycle CPU | cpu_alu, cpu_imem, cpu_dmem, cpu_regfile, cpu_control, cpu_datapath, cpu_top | 7 submodules |
| M05 | Control Unit | ctrl_phase01, ctrl_phase234, ctrl_top | 3 submodules |
| M06 | RISC-V Pipeline | rv_if_stage, rv_id_stage, rv_ex_stage, rv_mem_stage, rv_forward, rv_hazard, rv_top | 7 submodules |
| M07 | Cache | cache_arrays, cache_hit_logic, cache_ctrl, cache_top | 4 submodules |
| M08 | Arbiter | rr_ptr, rr_grant_logic, rr_top | 3 submodules |
| M09 | AES-128 | aes_sbox_lo, aes_sbox_hi, aes_sbox, aes_subbytes, aes_shiftrows, aes_gf_mul, aes_mixcolumns, aes_addroundkey, aes_keyschedule, aes_top | 10 submodules |
| M10 | SHA-256 | sha256_sigma, sha256_msg_sched, sha256_k_rom, sha256_compress, sha256_top | 5 submodules |
| M11 | CRC-32 | crc32_lfsr, crc32_ctrl, crc32_top | 3 submodules |
| M12 | FFT | fft_twiddle_rom, fft_butterfly, fft_stage1, fft_stage23, fft_top | 5 submodules |
| M13 | Matrix Multiply | mac_unit, mat_row, mat_top | 3 submodules |
| M14 | Bubble Sort | compare_swap, bubble_sort_fsm, bubble_sort_top | 3 submodules |

Total: 64 submodule generation jobs per model × 3 models = 192 total generation runs

## 3.3 Verification of Completeness

After all runs were complete, the following shell commands were used to confirm that no submodule was truncated (i.e., every generated .v file contains a proper endmodule statement):

# Check no files are missing endmodule (grep -L = files NOT containing the pattern)

$ grep -rL "endmodule" /scratch/soumyaj/vgen_project/verigen_out/2B/**/*.v

$ grep -rL "endmodule" /scratch/soumyaj/vgen_project/verigen_out/6B/**/*.v

$ grep -rL "endmodule" /scratch/soumyaj/vgen_project/verigen_out/16B/**/*.v

# If no output is printed, ALL files contain endmodule — zero truncations

Result: No modules were cut off after breaking them into submodules. All 64 × 3 = 192 generated files contained complete endmodule statements. This validates the submodule decomposition strategy.

# 4. HPC Environment Setup

## 4.1 Cluster Overview

All experiments were run on the university HPC cluster. The relevant compute resources used in this project are summarised below:

- Login node: hpc01 (used for job submission, file management, and environment setup)

- Compute nodes: gpu_h100_ partition — nodes equipped with NVIDIA H100 80GB SXM5 GPUs

- Storage: /scratch/soumyaj/ — high-performance parallel filesystem for model weights and outputs

- Scheduler: SLURM (Simple Linux Utility for Resource Management)

- Python environment: Python 3.10 venv with PyTorch + HuggingFace Transformers

## 4.2 First-Time Environment Setup (Step-by-Step)

The following steps were performed once on the login node to prepare the project environment. Any new user replicating this work should follow these steps in order.

### Step 1: Log into the HPC cluster

# Replace <username> with your actual username

$ ssh <username>@hpc01.university.edu

# Navigate to the scratch filesystem (do NOT use home directory for large files)

$ cd /scratch/$USER

$ mkdir vgen_project && cd vgen_project

### Step 2: Deactivate any active Conda environment

# If you see (base) or any conda env in your prompt, deactivate it first.

# Mixing conda and venv causes silent package conflicts.

$ conda deactivate

# Verify conda is deactivated (prompt should show no parenthesised env name)

$ echo $CONDA_DEFAULT_ENV   # Should print nothing

### Step 3: Create and activate a Python venv

# Create a virtual environment in the project directory

$ python3 -m venv venv

# Activate it

$ source venv/bin/activate

# Your prompt should now show: (venv) [username@hpc01 vgen_project]$

### Step 4: Install required Python packages

# Upgrade pip first

(venv) $ pip install --upgrade pip

# Install PyTorch for CUDA 12.1 (matches the H100 driver version on this cluster)

(venv) $ pip install torch torchvision torchaudio \

--index-url https://download.pytorch.org/whl/cu121

# Install HuggingFace Transformers and Accelerate

(venv) $ pip install transformers accelerate

# Verify GPU is visible

(venv) $ python -c "import torch; print(torch.cuda.is_available()); print(torch.cuda.get_device_name(0))"

# Expected output: True

# Expected output: NVIDIA H100 80GB HBM3

### Step 5: Upload project files

# From your local machine, use scp to upload your scripts

$ scp verigen_runner.py <username>@hpc01.university.edu:/scratch/<username>/vgen_project/

# Or use the HPC web portal file manager if SCP is restricted

## 4.3 CUDA Environment Errors and Fixes

A series of CUDA-related errors were encountered during initial job runs. These are documented here so that future users can resolve them quickly.

### Error 1: Missing Hardcoded CUDA Path

Symptom: Jobs fail with error: /usr/local/cuda-12.3: No such file or directory

Cause: The SLURM script contained a hardcoded path (export CUDA_HOME=/usr/local/cuda-12.3) that only existed on certain compute nodes. The gpu_h100_ nodes had a different CUDA installation path.

Fix: Replace the hardcoded versioned path with the universal symlink:

# In your SLURM script, change this:

# export CUDA_HOME=/usr/local/cuda-12.3   # WRONG - node-specific

# To this:

export CUDA_HOME=/usr/local/cuda           # CORRECT - universal symlink

export PATH=$CUDA_HOME/bin:$PATH

export LD_LIBRARY_PATH=$CUDA_HOME/lib64:$LD_LIBRARY_PATH

### Error 2: Module System Not Available on Compute Nodes

Symptom: module: command not found when running module load cuda inside batch job

Cause: The lmod module system on this cluster is configured for interactive login sessions only. Compute nodes running batch SLURM jobs do not source the module system initialisation scripts.

Fix: Do NOT use module load commands inside SLURM scripts. Use direct path exports instead (as shown in Error 1 fix above).

### Error 3: PyTorch/CUDA Driver Version Mismatch

Symptom: RuntimeError: CUDA error: no kernel image is available for execution on the device OR UserWarning: CUDA initialization failure

Cause: The pip-installed PyTorch was version 2.1.1+cu130 (compiled for CUDA 13.0). The H100 nodes have CUDA 12.x drivers installed. The driver version (12.x) is older than what the PyTorch binary was compiled for (13.0), so it cannot load the GPU kernels.

Fix: Uninstall the incompatible PyTorch and install the CUDA 12.1 build, which is universally accepted by CUDA 12.x drivers:

(venv) $ pip uninstall torch torchvision torchaudio -y

(venv) $ pip install torch torchvision torchaudio \

--index-url https://download.pytorch.org/whl/cu121

# Verify after installation

(venv) $ python -c "import torch; print(torch.__version__)"

# Expected: 2.x.x+cu121

## 4.4 Pre-Job Checklist

Before submitting any SLURM job, verify the following from the login node:

- Environment active: source venv/bin/activate (prompt shows (venv))

- CUDA check: python -c "import torch; print(torch.cuda.is_available())" prints True

- Script uploaded: ls -lh verigen_runner.py shows the file

- Output directory exists: mkdir -p verigen_out/2B verigen_out/6B verigen_out/16B

- Logs directory exists: mkdir -p logs

# 5. SLURM Job Submission

## 5.1 Understanding the SLURM Workflow

SLURM (Simple Linux Utility for Resource Management) is the job scheduler on the HPC cluster. Instead of running the Python script directly (which would tie up your terminal for hours and get killed if you disconnect), you submit a job script that SLURM queues and executes on a compute node automatically, even while you are offline.

The key concept used here is a SLURM Job Array: a single sbatch submission that creates multiple independent jobs (called array tasks), each running one submodule generation job. This is perfect for this project because each of the 64 submodule jobs is completely independent.

## 5.2 The SLURM Batch Script

Save the following as verigen_job.sh in your project directory:

#!/bin/bash

#SBATCH --job-name=verigen_          # Name shown in squeue

#SBATCH --partition=gpu_h100_        # GPU partition name (check with: sinfo)

#SBATCH --gres=gpu:1                 # Request 1 GPU per task

#SBATCH --cpus-per-task=4            # CPU cores for data loading

#SBATCH --mem=64G                    # RAM (model weights need ~25-48GB)

#SBATCH --time=01:15:00              # 75 min per task (37 min actual + buffer)

#SBATCH --array=0-63                 # Launch 64 tasks: indices 0 through 63

#SBATCH --output=logs/verigen_%A_%a.out   # stdout log per task

#SBATCH --error=logs/verigen_%A_%a.err    # stderr log per task

# ─── Environment setup ────────────────────────────────────────────────

# Navigate to project directory

cd /scratch/$USER/vgen_project

# Set CUDA paths (do NOT use 'module load' — it does not work in batch jobs)

export CUDA_HOME=/usr/local/cuda

export PATH=$CUDA_HOME/bin:$PATH

export LD_LIBRARY_PATH=$CUDA_HOME/lib64:$LD_LIBRARY_PATH

# Activate the Python virtual environment

source venv/bin/activate

# Tell HuggingFace to cache model weights in scratch (not home dir)

export HF_HOME=/scratch/$USER/.cache/huggingface

export TRANSFORMERS_CACHE=/scratch/$USER/.cache/huggingface

# ─── Run the submodule at the SLURM array task index ──────────────────

# $SLURM_ARRAY_TASK_ID is automatically set to 0, 1, 2 ... 63 by SLURM

echo "[START] Task ID: $SLURM_ARRAY_TASK_ID on $(hostname) at $(date)"

python verigen_runner.py \

--model 6B \

--output_dir ./verigen_out \

--idx $SLURM_ARRAY_TASK_ID

echo "[END] Task $SLURM_ARRAY_TASK_ID finished at $(date)"

Important: Change --model to 2B, 6B, or 16B as needed. For the 16B model, increase --mem to 80G.

## 5.3 Submitting the Job

# Make the script executable

$ chmod +x verigen_job.sh

# Submit to SLURM

$ sbatch verigen_job.sh

# SLURM will respond with: Submitted batch job 159592

# (your job ID will be different)

## 5.4 Monitoring Your Jobs

# ─── Check queue status ───────────────────────────────────────────────

$ squeue -u $USER

# Columns explained:

#  JOBID   = Job array ID (e.g., 159592_[0-63]) or individual task ID

#  PARTITION = Queue/partition name

#  NAME    = Job name

#  USER    = Your username

#  ST      = Status: R=Running, PD=Pending, CG=Completing

#  TIME    = Elapsed time

#  NODELIST(REASON) = Which node, or why it's pending

# ─── Common pending reasons ───────────────────────────────────────────

# (QOSMaxCpuPerUserLimit) = You've hit your GPU quota; tasks are queued

#   and will start automatically as running tasks finish. NORMAL.

# (Resources)           = No free GPUs right now; will start when available

# (Dependency)          = Job waiting for another job to finish first

# ─── Check a specific job ─────────────────────────────────────────────

$ squeue -j 159592

# ─── Watch the queue live (refreshes every 5 seconds) ────────────────

$ watch -n 5 squeue -u $USER

# ─── If the queue is empty, all tasks have finished ───────────────────

$ squeue -u $USER   # prints nothing = all done

## 5.5 Checking Results After Jobs Complete

# Count total .v files generated (expect 64 per model)

$ find verigen_out/6B/ -name '*.v' | wc -l

# List all files by model and module

$ for d in 2B 6B 16B; do

>   echo '===== '$d' ====='

>   find verigen_out/$d -type f -name '*.v' | sort

> done

# Verify no files are truncated (missing endmodule)

$ grep -rL 'endmodule' verigen_out/6B/**/*.v

# (No output = all files are complete)

# Read a specific generated file

$ cat verigen_out/6B/m01_alu/alu_top.v

# Check error logs for any crashes (HuggingFace warnings are normal)

$ grep -i 'error\|traceback\|exception' logs/verigen_*.err

## 5.6 Cancelling Jobs

# Cancel a specific job

$ scancel 159592

# Cancel all YOUR jobs

$ scancel -u $USER

# Cancel only pending jobs (leave running jobs alone)

$ squeue -u $USER -h -t PD -o '%i' | xargs scancel

## 5.7 GPU and Cluster Resource Commands

# ─── Check available GPU partitions ──────────────────────────────────

$ sinfo -o '%.12P %.10a %.12l %.6D %N'

# ─── Check GPU usage on a specific node ──────────────────────────────

$ srun --partition=gpu_h100_ --gres=gpu:1 --pty nvidia-smi

# ─── Check your storage quota on scratch ─────────────────────────────

$ du -sh /scratch/$USER/

# ─── Check available disk space on scratch ───────────────────────────

$ df -h /scratch/

# ─── Check job efficiency after completion ───────────────────────────

$ seff <JOBID>    # Shows CPU/memory efficiency for a completed job

# ─── Show detailed job history ────────────────────────────────────────

$ sacct -u $USER --format=JobID,JobName,Elapsed,State,ExitCode -X

## 5.8 Timing Breakdown (Per Task)

Each individual array task (one submodule on one model) follows this timing breakdown on the H100:

- Model loading (12GB from /scratch to GPU VRAM): ~31-35 minutes

- Tokenisation + generation: ~2 minutes

- Total per task: ~37 minutes

Important note on QOSMaxCpuPerUserLimit: The HPC cluster enforces a per-user GPU quota. If you submit 64 tasks and only 1 GPU is granted at a time, the tasks execute serially: Task 0 runs, finishes, Task 1 starts, and so on. With 64 tasks × ~37 minutes, the full array takes approximately 40 hours to complete. This is completely normal — SLURM handles everything automatically. You can safely close your terminal and return the next day.

# 6. Results: 2B Model on 14 Monolithic Modules

The 2B model was first tested on all 14 modules in their original monolithic form (a single long prompt per module, no decomposition). This baseline run reveals the raw capabilities and limitations of the smallest model under real token-pressure conditions.

| # | Module | Time (H100) | Max Tokens | Output Quality |
| --- | --- | --- | --- | --- |
| 1 | N-bit ALU | 32s | 2000 | Partial — mixed comb/sequential; extra regfile/testbench appended |
| 2 | Register File | 36s | 2000 | Partial — incorrect x0 handling; extra mux/testbench appended |
| 3 | UART Transmitter | 1m 10s | 1700 | Incomplete — token limit hit mid-generation; extra uart_rx appended |
| 4 | Multi-Cycle CPU | 1m 7s | 1700 | Incomplete — output truncated; FSM logic fragmented |
| 5 | Control Unit | 1m 11s | 1700 | Partial — only phase 0 and 2 covered; datapath appended with errors |
| 6 | RISC-V Pipeline | 56s | 1500 | Incomplete — only register declarations; no logic |
| 7 | Direct-Mapped Cache | 1m 11s | 1700 | Incorrect — wrong architecture; no hit/miss logic |
| 8 | Round-Robin Arbiter | 1m 8s | 1700 | Incorrect — wrong sensitivity list; undefined signal prio_bit_i |
| 9 | AES-128 Core | 1m 4s | 1600 | Incomplete — only partial S-box LUT (stopped at 0x55); no round logic |
| 10 | SHA-256 Core | 1m 5s | 1500 | Incorrect — wrong data types (64-bit not 32-bit); no compression rounds |
| 11 | CRC-32 Engine | 1m 11s | 1700 | Partial — LFSR logic present but wrong bit-ordering; unrelated AV-ST module appended |
| 12 | 8-Point FFT | 1m 0s | 1500 | Incorrect — no butterfly logic; just copies inputs to outputs |
| 13 | 16×16 MatMul | 46s | 1700 | Incorrect — wrong generate indexing; drive conflicts; no MAC logic |
| 14 | Bubble Sort | 1m 10s | 1700 | Partial — FSM structure present but swap logic wrong; extra mux module appended |

Legend: Red = Incorrect/Incomplete    Amber = Partial

## 6.1 Key Observations from 2B Baseline

### Context Pollution / Module Bleeding

In 9 out of 14 runs, the model appended entirely unrelated modules after completing (or failing to complete) the target module. Examples include: a regfile appended after the ALU, a uart_rx module after the uart_tx, and a large Altera video decoder appended after CRC-32. This behaviour stems from the model having learned patterns from multi-module Verilog training files and struggling to terminate cleanly after a single module.

### Abrupt Truncation

For 5 modules (UART, multi-cycle CPU, RISC-V pipeline, AES, SHA-256), the output was simply cut off mid-code because the 2048-token budget ran out. The generated Verilog ended without endmodule, sometimes mid-always-block or mid-assign-statement.

### Semantic Errors in Algorithmic Modules

Modules requiring precise mathematical constants (CRC-32 polynomial, AES S-box, FFT twiddle factors, SHA-256 K values) were almost universally wrong. The model could reproduce comment headers and port declarations accurately but failed to implement the underlying mathematical operations.

# 7. Error Taxonomy

All observed errors across all generated outputs (2B monolithic + 2B/6B/16B submodules) are classified into the following categories. This taxonomy provides a quantitative framework for comparing model quality across module types and model sizes.

| Code | Name | Description | Primary Modules |
| --- | --- | --- | --- |
| F1 | Syntax Error | Elaboration fails: undeclared signals, wrong types, missing semicolons. Vivado/iverilog rejects at compile time. | All (2B monolithic) |
| F2 | Wrong Constants | Wrong polynomial, S-box values, twiddle factors. Code compiles and simulates but produces wrong output. | CRC-32, AES, FFT |
| F3 | Structural Incomplete | Required sub-modules or always blocks missing entirely. Module declaration present, body missing. | AES, FFT, CPUs |
| F4 | Non-Synthesisable | for-loops with variable bounds, initial blocks, $display/$monitor. Vivado synth_design rejects. | Sort, MatMul, ALU |
| F5 | Functional Mismatch | Compiles and simulates but produces wrong output. Logic error in FSM, wrong operation order, wrong signal routing. | All (16B, late prompts) |
| F6 | Bit-Width Error | Signal widths inconsistent; truncation/sign-extension causes wrong computation. | SHA, FFT, MAC, ALU |
| PASS | Correct | All Vivado testbench assertions pass. Module is synthesisable and functionally correct. | — |

## 7.1 Detailed Error Descriptions

### F1 — Syntax Error

These are the most basic failures. Vivado elaboration or iverilog compilation fails with messages such as:

# Common Vivado/iverilog F2 error messages:

ERROR: [Synth 8-439] module 'alu_addsub' not found

ERROR: [Synth 8-3352] Sequential element is not in an always block

syntax error near 'endmodule'

undefined variable 'prio_bit_i'

F1 errors are most common in 2B monolithic runs where complex modules were attempted in a single generation. The submodule decomposition strategy significantly reduces F1 occurrence because each submodule has a simpler, more focused task.

### F2 — Wrong Constants

The model correctly generates the module structure and port connections, but embeds wrong numerical constants. Critical examples observed:

- CRC-32: Polynomial bit mask transposed or shifted; LFSR feedback taps incorrect

- AES S-box: Values partially correct in early entries but diverge after index 0x50

- FFT twiddle factors: Q1.15 fixed-point values wrong sign or wrong magnitude

- SHA-256 K ROM: Values derived incorrectly from block_in instead of hardcoded cube-root constants

### F3 — Structural Incomplete

The module declaration and port list are generated correctly, but key logic sections are absent. This differs from truncation (F1) in that the endmodule keyword is present — the model simply skipped generating the actual logic. Commonly observed in the AES round function (no key schedule generated) and FFT (twiddle ROM present, butterfly logic absent).

### F4 — Non-Synthesisable

The generated code is valid Verilog for simulation but cannot be synthesised to hardware by Vivado synth_design. Common patterns:

- for (integer i = 0; ...) — For-loops with non-constant bounds in always blocks

- initial begin ... end — Initialisation blocks (simulation only)

- $display, $monitor, $dumpfile — System tasks in RTL code (not synthesisable)

- assign to a reg variable — Continuous assignment to a registered output (both assign and always block driving same signal)

### F5 — Functional Mismatch

The module compiles, elaborates, and simulates without errors, but the output values are wrong when validated against a testbench with known golden vectors. This is the most insidious class of error because automated syntax checking passes. Examples:

- Round-Robin Arbiter: Grant output uses combinational (not registered) update; priority pointer not advanced correctly

- Bubble Sort FSM: Pass and step counter logic inverted; sort terminates early

- Register File: x0 hardwiring logic incorrect; write to x0 not suppressed under all conditions

### F6 — Bit-Width Error

Signal width mismatches cause incorrect computation. Examples:

- SHA-256: Working variables declared as 64-bit (reg [63:0]) instead of 32-bit (reg [31:0]), causing accumulation to overflow wrong boundary

- ALU: carry_out computed on wrong bit width, giving wrong borrow for SUB operation

- MAC unit: Accumulator not wide enough to hold sum of 16 16-bit products (needs 32-bit, generated as 16-bit)

# 8. Testbench Generation Strategy

Testbenches for all 64 submodules and 14 top-level modules must be written by an external LLM (GPT-4o, Gemini Pro, or Claude) — NOT by VeriGen itself. This is intentional: VeriGen is the system under evaluation, and having it generate its own testbenches would create circular validation.

## 8.1 Which LLM to Use for Testbenches

Any of the following are suitable. Use the same LLM consistently within a single evaluation run:

- Claude: Recommended for structured Verilog testbench generation. Strong at register-transfer-level logic descriptions.

- GPT-4o: Good for arithmetic modules and cryptographic verification logic.

- Gemini Pro: Adequate for basic FSM and datapath testbenches.

## 8.2 What a Good Testbench Must Cover

For each submodule or top-level module, the testbench should include:

- Reset sequence: Assert synchronous reset for at least 5 clock cycles before applying stimulus

- Golden reference values: Hard-coded expected outputs for at least 8 distinct input combinations

- $display assertions: Print PASS/FAIL for each test vector

- Edge cases: Zero inputs, all-ones inputs, maximum value inputs

- Functional completeness: Every FSM state must be visited at least once

- Self-checking: Testbench exits with $finish only after all assertions are evaluated

## 8.3 Prompt Template for Testbench Generation

Use the following prompt template when asking any LLM to generate a testbench. Paste the generated Verilog module into <PASTE_MODULE_HERE>:

You are a Verilog RTL verification engineer.

Write a complete, self-checking Verilog testbench for the following module.

REQUIREMENTS:

1. Use Verilog 2001 syntax only (no SystemVerilog).

2. Declare all signals as reg or wire, not logic.

3. Generate a 10ns clock (50MHz equivalent).

4. Assert synchronous reset for 5 cycles at the start.

5. Apply at least 8 distinct test vectors covering:

- Normal operation

- Edge cases (zero, all-ones, max-value)

- All FSM states if applicable

6. After each test vector, print PASS or FAIL using $display.

7. End with $finish and do NOT use $stop.

8. Do NOT include any generate blocks or SystemVerilog constructs.

9. The testbench module name must be tb_<module_name>.

MODULE TO TEST:

<PASTE_MODULE_HERE>

## 8.4 Testbench File Naming Convention

To keep the project organised, save all testbenches in a parallel tb/ directory structure mirroring the verigen_out/ hierarchy:

# Directory structure

vgen_project/

|-- verigen_out/          # Generated RTL from VeriGen

|   |-- 2B/

|   |   |-- m01_alu/

|   |   |   |-- alu_addsub.v

|   |   |   |-- alu_top.v

|   |   |-- m02_regfile/

|   |   |-- ...

|   |-- 6B/

|   |-- 16B/

|

|-- tb/                   # Testbenches (written by external LLM)

|   |-- m01_alu/

|   |   |-- tb_alu_addsub.v

|   |   |-- tb_alu_top.v

|   |-- m02_regfile/

|   |-- ...

|

|-- verigen_runner.py

|-- verigen_job.sh

## 8.5 Per-Module Testbench Summary

Below is a concise specification for what each module's testbench must verify:

| Module | Key Verification Checks |
| --- | --- |
| ALU | ADD/SUB carry flag; AND/OR/XOR truth table; SHL carry; zero flag when result==0; synchronous reset clears all outputs |
| Register File | x0 always reads 0 regardless of write; WAR (write-after-read) old value returned; reset zeroes all 32 registers; both read ports independent |
| UART TX | baud tick period = CLK_FREQ/BAUD_RATE; start bit is LOW; stop bit is HIGH; 8 data bits LSB first; busy deasserts after stop bit |
| Multi-Cycle CPU | ADD instruction executes in 5 phases; LW reads correct memory address; BEQ branches when rs1==rs2; SW writes to correct address |
| Control Unit | Every opcode+phase combination produces correct control signals; default all-zero for undefined inputs |
| RISC-V Pipeline | Forwarding resolves RAW hazard in 1 cycle; load-use stall inserts NOP; BEQ flushes pipeline on taken branch |
| Cache | Read hit returns correct data with hit=1; read miss returns mem_rdata and updates tag; write updates cache and drives mem_we=1 |
| Arbiter | Round-robin rotation: after granting bit i, next grant is bit i+1; grant=0 when req=0; starvation test: all bits requested |
| AES-128 | S-box correctness: sbox[0x00]=0x63, sbox[0x53]=0xED; ShiftRows row offset; AddRoundKey XOR identity test |
| SHA-256 | sigma0/sigma1 rotation values; Ch and Maj logic; first 8 bytes of known test vector hash match golden reference |
| CRC-32 | Known message 'Hello' produces CRC 0xF7D18982; init resets LFSR to 0xFFFFFFFF; final XOR applied correctly |
| FFT | W^0 butterfly: A'=A+B, B'=A-B; output for DC input equals sum of all inputs; output valid_out asserts 3 cycles after valid_in |
| Matrix Multiply | C[0][0] = sum of A[0][k]*B[k][0] for k=0..15; identity matrix multiplication returns original; done asserts after 16*16 cycles |
| Bubble Sort | 8-element sort: [5,1,4,2,8,3,7,6] produces [1,2,3,4,5,6,7,8]; already-sorted input completes without error; single-element array trivially passes |

9. Integration: Submodules to Top-Level Modules

## 9.1 Why Integration is Required

VeriGen generated each submodule as an independent .v file. To run functional simulation and synthesis on the complete system (e.g., a full RISC-V pipeline or complete AES core), the submodule files must be integrated into a single Verilog project where the top-level module instantiates its children.

This integration step also reveals structural errors (F3): if a submodule's port list does not match what the top-level module expects, there is a mismatch between what VeriGen generated and what the design requires.

## 9.2 Integration Strategy

For each of the 14 benchmark modules, follow this procedure:

- Step 1 — Collect all submodule .v files for a given module (e.g., all 10 AES submodules into a single working directory)

- Step 2 — Inspect port interfaces of each generated submodule. Compare against the expected interface defined in the verigen_runner.py prompt. Note any mismatches.

- Step 3 — Write or generate an integration wrapper if the top-level module's instantiations need adjusting to match the actual generated port names.

- Step 4 — Compile all files together using iverilog or add them as source files in Vivado.

- Step 5 — Run the top-level testbench and observe pass/fail results.

## 9.3 Integration Using iverilog (Free, Command-Line)

iverilog is a free open-source Verilog simulator. It is already available on most HPC Linux nodes. Use it for a quick syntax and elaboration check before opening Vivado.

# ─── Install iverilog if not present ────────────────────────────────

$ sudo apt-get install iverilog   # Ubuntu/Debian

$ module load iverilog            # Some HPC clusters have it as a module

# ─── Single file compile + simulate ─────────────────────────────────

$ iverilog -o sim_alu verigen_out/6B/m01_alu/alu_top.v tb/m01_alu/tb_alu_top.v

$ vvp sim_alu

# ─── Multi-file compile (all submodules + testbench) ─────────────────

$ iverilog -o sim_aes \

verigen_out/6B/m09_aes/aes_sbox.v \

verigen_out/6B/m09_aes/aes_subbytes.v \

verigen_out/6B/m09_aes/aes_shiftrows.v \

verigen_out/6B/m09_aes/aes_gf_mul.v \

verigen_out/6B/m09_aes/aes_mixcolumns.v \

verigen_out/6B/m09_aes/aes_addroundkey.v \

verigen_out/6B/m09_aes/aes_keyschedule.v \

verigen_out/6B/m09_aes/aes_top.v \

tb/m09_aes/tb_aes_top.v

$ vvp sim_aes

# ─── Batch compile all modules for a given model ─────────────────────

$ for mod in m01_alu m02_regfile m03_uart m04_cpu m05_ctrl m06_riscv \

>             m07_cache m08_arbiter m09_aes m10_sha256 m11_crc32 \

>             m12_fft m13_matmul m14_sort; do

>   echo '--- Compiling '$mod' ---'

>   iverilog -o sim_${mod} verigen_out/6B/${mod}/*.v tb/${mod}/tb_*_top.v 2>&1

> done

## 9.4 Integration Using Vivado (Full Synthesis + Simulation)

Vivado is the primary EDA tool for full synthesis and implementation. The following steps cover creating a project, adding all generated files, and running synthesis.

### Step 1: Create a Vivado Project

Open Vivado and create a new RTL project. Set the target device to a Xilinx FPGA (e.g., xc7a35ticsg324-1L for an Artix-7, or xcvu9p for a Virtex UltraScale+).

### Step 2: Add All Submodule Source Files

# In the Vivado Tcl Console, add all .v files for one model+module combination:

# Add all 6B AES submodules

add_files -norecurse [glob verigen_out/6B/m09_aes/*.v]

# Add the testbench separately as a simulation source

add_files -fileset sim_1 -norecurse [glob tb/m09_aes/*.v]

# Set the top-level module for synthesis

set_property top aes_top [current_fileset]

# Set the top-level module for simulation

set_property top tb_aes_top [get_filesets sim_1]

### Step 3: Run Elaboration (Syntax + Structural Check)

# From Vivado Tcl Console:

synth_design -rtl -name rtl_1

# Or from GUI: Flow -> Elaborated Design

# Errors here = F2 (Syntax) or F4 (Structural Incomplete)

### Step 4: Run Simulation

# From Vivado Tcl Console:

launch_simulation

run all

# Check the Tcl Console for PASS/FAIL prints from $display statements

# Mismatches here = F6 (Functional Mismatch), F3 (Wrong Constants), or F7 (Bit-Width)

### Step 5: Run Synthesis

# From Vivado Tcl Console:

synth_design -top aes_top -part xcvu9p-flgb2104-2-i

# Errors here = F5 (Non-Synthesisable)

# A clean synthesis with no critical warnings = PASS tier

# 10. Quantifying and Scoring the Results

## 10.1 Evaluation Pipeline

The following pipeline should be applied to every generated .v file across all 64 submodules and 3 model sizes (192 files total):

- Stage 1 — Truncation Check: grep -L 'endmodule' — classify as F3 (Structural Incomplete) if missing

- Stage 2 — Syntax Check: iverilog -t null — any compilation error is F1 (Syntax Error)

- Stage 3 — Elaboration Check: Vivado synth_design -rtl — structural errors that pass iverilog but fail Vivado elaboration

- Stage 4 — Synthesis Check: synth_design — non-synthesisable constructs are F4

- Stage 5 — Simulation: Run testbench with known golden vectors — wrong output is F5, F2, or F6 depending on root cause

- Stage 6 — Root Cause Classification: Manually inspect failing outputs and assign F1-F6 or PASS label

## 10.2 Automated Batch Evaluation Script

The following bash script automates Stages 1 and 2 across all generated files:

#!/bin/bash

# evaluate_batch.sh — Runs iverilog syntax check on all generated .v files

PASS=0; FAIL=0; TOTAL=0

for model in 2B 6B 16B; do

for vfile in verigen_out/${model}/**/*.v; do

TOTAL=$((TOTAL + 1))

# Stage 1: truncation check

if ! grep -q 'endmodule' "$vfile"; then

echo "F4 TRUNCATED: $vfile"

FAIL=$((FAIL + 1))

continue

fi

# Stage 2: iverilog syntax check

if iverilog -t null "$vfile" 2>/dev/null; then

echo "SYNTAX_OK: $vfile"

PASS=$((PASS + 1))

else

echo "F2 SYNTAX_ERR: $vfile"

FAIL=$((FAIL + 1))

fi

done

done

echo '==============================='

echo "Total: $TOTAL  Pass: $PASS  Fail: $FAIL"

echo "Syntax Pass Rate: $(echo 'scale=1; '$PASS'*100/'$TOTAL | bc)%"

## 10.3 Expected Results Summary Table

After running the full evaluation pipeline, populate this table. The structure is provided here as a template — fill in actual pass/fail counts after running the testbenches.

| Module | 2B Result | 6B Result | 16B Result | Dominant Error |
| --- | --- | --- | --- | --- |
| ALU (alu_top) | F6 | TBD | TBD | F5 (assign to reg) |
| Register File | F6 | TBD | TBD | F6 (x0 logic wrong) |
| UART TX | F4 | TBD | TBD | F4 (token truncation) |
| Multi-Cycle CPU | F4 | TBD | TBD | F4 (token truncation) |
| Control Unit | F6 | TBD | TBD | F4 (missing phases) |
| RISC-V Pipeline | F4 | TBD | TBD | F4 (only declarations) |
| Direct-Mapped Cache | F2 | TBD | TBD | F2 (wrong architecture) |
| Round-Robin Arbiter | F2 | TBD | TBD | F2 (undefined signal) |
| AES-128 | F4 | TBD | TBD | F4 (S-box incomplete) |
| SHA-256 | F1 | TBD | TBD | F1 (64-bit vars) |
| CRC-32 | F3 | TBD | TBD | F3 (wrong polynomial bit order) |
| FFT | F6 | TBD | TBD | F4 (no butterfly logic) |
| Matrix Multiply | F5 | TBD | TBD | F5 (assign to reg in generate) |
| Bubble Sort | F6 | TBD | TBD | F6 (wrong FSM transitions) |

Note: TBD = To Be Determined after running testbenches. Column '2B Result' reflects 2B monolithic runs. Submodule runs (2B/6B/16B decomposed) should be scored independently and may show improved results.

# 11. Complete HPC Command Reference

This section serves as a quick-reference cheat sheet for all commands needed throughout the project lifecycle.

## 11.1 Environment Management

# Login to HPC

$ ssh <username>@hpc01.university.edu

# Deactivate conda (ALWAYS do this first)

$ conda deactivate

# Create venv (first time only)

$ python3 -m venv /scratch/$USER/vgen_project/venv

# Activate venv (every session)

$ source /scratch/$USER/vgen_project/venv/bin/activate

# Deactivate venv

$ deactivate

# Check Python version

$ python --version

# Check installed packages

$ pip list | grep -E 'torch|transformers|accelerate'

## 11.2 Job Submission and Monitoring

# Submit job array

$ sbatch verigen_job.sh

# Check queue

$ squeue -u $USER

# Watch queue live

$ watch -n 5 squeue -u $USER

# Cancel all jobs

$ scancel -u $USER

# Check job efficiency

$ seff <JOBID>

# View job history

$ sacct -u $USER --format=JobID,JobName,Elapsed,State,ExitCode -X

# Interactive GPU session (for testing scripts)

$ srun --partition=gpu_h100_ --gres=gpu:1 --mem=64G --time=00:30:00 --pty bash

## 11.3 File Management

# Count generated .v files

$ find verigen_out/ -name '*.v' | wc -l

# List files by model

$ find verigen_out/6B -name '*.v' | sort

# Check for truncated files (missing endmodule)

$ grep -rL 'endmodule' verigen_out/**/*.v

# Check error logs

$ grep -i 'error\|traceback' logs/verigen_*.err

# Archive outputs

$ tar -czf verigen_results_$(date +%Y%m%d).tar.gz verigen_out/

# Check scratch disk usage

$ du -sh /scratch/$USER/

# Transfer results to local machine

$ scp -r <username>@hpc01.university.edu:/scratch/<username>/vgen_project/verigen_out/ ./local_results/

## 11.4 iverilog Commands

# Syntax check only (no simulation)

$ iverilog -t null design.v

# Compile and simulate

$ iverilog -o sim_out design.v testbench.v && vvp sim_out

# Compile multiple files

$ iverilog -o sim_out *.v tb/tb_top.v

# Generate VCD waveform

$ vvp sim_out   # (testbench must contain $dumpfile and $dumpvars calls)

$ gtkwave dump.vcd &

## 11.5 Vivado Tcl Commands

# Start Vivado in batch mode

$ vivado -mode batch -source run_synth.tcl

# In Vivado Tcl Console:

# Add all source files

add_files -norecurse [glob verigen_out/6B/m01_alu/*.v]

# Add simulation files

add_files -fileset sim_1 -norecurse [glob tb/m01_alu/*.v]

# Set top-level module

set_property top alu_top [current_fileset]

# Elaborate (check syntax + structure)

synth_design -rtl -name check_1

# Run synthesis

synth_design -top alu_top -part xc7a35ticsg324-1L

# Launch simulation

launch_simulation

run all

# Read synthesis report

report_utilization

report_timing_summary

# 12. Summary and Next Steps

## 12.1 What Was Accomplished

The following work has been completed and is documented in this report:

- Defined 14 novel RTL benchmark modules spanning logic, architecture, cryptography, signal processing, and algorithms — not previously tested in the original VeriGen paper

- Designed a hierarchical decomposition strategy that breaks all 14 modules into 64 self-contained submodules, each fitting comfortably within the 2048-token context window

- Implemented the VeriGen batch runner (verigen_runner.py) with SLURM array job support for fully automated overnight execution on the HPC cluster

- Solved three distinct HPC environment issues (hardcoded CUDA path, broken module system, PyTorch/driver mismatch) and documented fixes for future users

- Executed the full generation run on NVIDIA H100 GPUs: 2B model on 14 monolithic modules + all 3 models (2B/6B/16B) on all 64 submodules = 192 total generation jobs

- Verified zero truncations in the submodule decomposition run using grep -L 'endmodule' — all 192 files are complete

- Defined a 7-tier error taxonomy (F1–F6 + PASS) for classifying generated RTL quality

## 12.2 Remaining Work (Next Steps)

The following steps are required to complete the evaluation:

- Generate testbenches for all 64 submodules and 14 top-level modules using an external LLM (Claude, GPT-4o, or Gemini)

- Run iverilog batch evaluation using the evaluate_batch.sh script to score all 192 files on Stages 1-2

- Run Vivado elaboration and simulation for all 192 files to score Stages 3-5

- Populate the results summary table (Section 10.3) with actual pass/fail counts for all three model sizes

- Compare model sizes: Does 6B significantly improve over 2B? Does 16B further improve? Quantify the scaling benefit.

- Write final analysis report presenting the error distribution by module category and model size, with conclusions about when LLM-based RTL generation is and is not reliable

## 12.3 Key Findings (Preliminary)

Based on results to date:

- Token ceiling is the primary bottleneck — not model quality — for 7 of the 14 modules when tested as monolithic prompts

- Submodule decomposition eliminates truncation — zero F3 truncation errors across all 192 submodule files

- Semantic/algorithmic errors persist — AES, SHA-256, FFT, CRC-32 consistently produce wrong constants or wrong algorithm implementations regardless of model size

- Simple FSM-based modules (UART, arbiter, sort) are most amenable to LLM generation; complex mathematical cores are least amenable

- Context pollution (extra appended modules) is observed in 9/14 monolithic runs but is significantly reduced in submodule runs due to shorter, more focused prompts

# Appendix A: verigen_runner.py Key Parameters

The verigen_runner.py script drives all submodule generation. Key configurable parameters:

| Parameter | Default Value | Description |
| --- | --- | --- |
| max_length | 2048 | Hard architectural limit; do not increase |
| temperature | 0.2 | Low value = deterministic outputs; increase to 0.7 for more variety |
| top_p | 0.95 | Nucleus sampling cutoff |
| do_sample | True | Enable stochastic sampling |
| --model | 2B | Model size: 2B or 6B (16B requires separate script) |
| --output_dir | ./verigen_outputs | Root directory for all generated .v files |
| --idx | None | If set, runs only the single submodule at this index (used by SLURM) |
| --list | (flag) | Prints all 64 submodule indices and their prompt previews |

# Appendix B: VeriGen Model Reference

| Size | HuggingFace Model ID | VRAM Required | Notes |
| --- | --- | --- | --- |
| 2B | shailja/fine-tuned-codegen-2B-Verilog | ~10 GB | Fastest; good for submodules |
| 6B | shailja/fine-tuned-codegen-6B-Verilog | ~25 GB | Balanced quality/speed |
| 16B | shailja/fine-tuned-codegen-16B-Verilog | ~45 GB | Best quality; slowest load |

All models share the 2048-token context limit. All weights are freely available from HuggingFace Hub: huggingface.co/shailja

End of Report