HierRTLBench

Evaluating LLM-Based Verilog RTL Generation Under Context-Window Constraints

# Abstract

HierRTLBench (Hierarchical Register-Transfer Level Benchmark) is a benchmark suite and evaluation framework for measuring how well large language models (LLMs) can automatically write Verilog RTL. Existing benchmarks largely test small, isolated circuit pieces and were not designed to surface the failure modes that emerge when token-budget constraints interact with realistic, hierarchically complex hardware designs.

The work contributes: (1) a benchmark suite spanning processors, caches, cryptographic cores, DSP units, and communication interfaces; (2) a hierarchy-aware decomposition workflow that breaks large designs into context-safe submodule prompts (≤120 tokens each), reducing output truncation during generation while keeping per-query inference cost bounded; (3) an automated multi-layer verification pipeline built on the Icarus Verilog open-source simulator, showing that reductions in truncation do not translate into corresponding gains in end-to-end functional pass rates; and (4) a six-class RTL error taxonomy evaluated across VeriGen 2B, 6B, and 16B parameter models, where the 6B model collapses to a 0% pass rate.

Syntax pass rates across submodule runs are 44% (2B), 0% (6B), and 54% (16B); end-to-end functional pass rates are 30%, 0%, and 28% respectively.

# 1. Introduction

HDL development remains labor-intensive, expensive, and error-prone; a single processor design can require months of expert engineering. Verilog RTL describes how data moves between storage elements cycle by cycle, and LLMs fine-tuned on Verilog corpora have shown promise as automated RTL writers. Three structural limitations, however, prevent practical use:

- Token-window bottleneck: an LLM's context window covers both prompt and generation. Open-source models such as VeriGen inherit CodeGen's hard 2048-token limit. A 5-stage pipelined RISC-V prompt alone consumes ≈550 tokens, leaving ≈1498 for generation, while a complete processor needs ≈4000–6000 tokens — the model runs out of space and silently produces broken, truncated output.

- Shallow benchmarks: existing suites such as VerilogEval test only simple, isolated components (e.g. a single multiplexer), leaving the failure modes of realistic hierarchical designs (multi-stage pipelines, cryptographic state machines, cache controllers) largely uncharacterized.

- No systematic error analysis: published evaluations report pass/fail percentages without classifying why failures occur, and ad hoc, unpublished internal criteria make results incomparable across models and studies.

The central hypothesis: if the context window is the primary bottleneck, decomposing large designs into submodule prompts small enough to fit inside the window should close the functional gap in most cases. HierRTLBench tests exactly this hypothesis. Every submodule prompt stays ≤120 tokens; an automated pipeline compiles every generated file, classifies every error under a six-class taxonomy, and runs functional tests where compilation succeeds.

Across 150 verification runs, the key findings are: (i) the 6B model is catastrophically worse than 2B, with a 0% functional pass rate; (ii) larger models do not automatically improve results; (iii) token-budget pressure is a likely contributing factor in roughly 42.6% of observed outputs, based on truncation rates even after decomposition, though causal confirmation needs further controlled experiments; and (iv) complex modules — AES encryption, FFT, pipelined RISC-V — resist all model sizes.

# 2. Related Work

VeriGen trains five CodeGen model variants (345M–16B parameters) on 400MB of Verilog and evaluates on 17 hand-designed plus 164 simple problems. VerilogEval extends evaluation to 156 problems with automated simulation. RTLCoder shows a curated 7B model can match GPT-3.5-turbo on hardware tasks. GPT4AIGChip explores LLM-driven AI accelerator design automation. All of these share a limitation: benchmarks of single, simple modules that miss the hierarchical complexity of real chip design.

ROME introduces multi-step hierarchical prompting for chip design; HiVeGen extends this with code retrieval and design-space exploration (reporting 45.24% runtime savings over direct GPT generation) but targets large closed-source models rather than constrained open-source ones. RTLLM provides 30 benchmark designs; ChipChat explores conversational RTL design at the CPU level. RTLFixer addresses the complementary problem of automatically correcting syntax errors post-generation via RAG and ReAct prompting, correcting 98.5% of compile-time errors on VerilogEval-syntax, and finds roughly 55% of LLM-generated Verilog errors are syntax-related — consistent with the F1 rates observed here. HierRTLBench is orthogonal to all of these: it classifies error types across hierarchically complex designs before any repair stage, exposing failure modes (semantic mismatch, FSM sequencing errors, cryptographic constant hallucination) that survive syntax correction and that no repair framework has addressed at this complexity level.

# 3. HierRTLBench Design

## 3.1 Module Selection

HierRTLBench spans 14 RTL modules across 7 hardware design categories, covering the full range from simple combinational logic to complex pipelined processors and cryptographic engines. Modules were selected to mirror the complexity diversity of benchmark suites such as SPEC CPU2017 and MiBench when translated to equivalent hardware workloads.

| ID | Module | Category | # Submodules |
| --- | --- | --- | --- |
| M01 | Param. N-bit ALU | Datapath | 4 |
| M02 | 32-Entry Register File | Memory | 1 |
| M03 | UART TX (8-N-1) | Communication | 3 |
| M04 | Multi-Cycle RISC CPU | CPU | 6 |
| M05 | Hardwired Control Unit | Control | 2 |
| M06 | 5-Stage Pipelined RISC-V CPU | CPU | 6 |
| M07 | Direct-Mapped Cache | Memory | 3 |
| M08 | Round-Robin Arbiter | Interconnect | 2 |
| M09 | AES-128 Encryption Core | Cryptography | 9 |
| M10 | SHA-256 Hash Core | Cryptography | 4 |
| M11 | CRC-32 Engine (IEEE 802.3) | Error Detection | 2 |
| M12 | 8-Point Fixed-Point FFT | DSP | 4 |
| M13 | 16×16 Matrix Multiplier | DSP/ML | 2 |
| M14 | Hardware Bubble Sort | Algorithm | 2 |

Table 1. HierRTLBench module summary — 14 modules, 7 categories, 50 submodules total.

## 3.2 Submodule Decomposition

Each module is decomposed into independently generated RTL submodules following standard hardware design practice: leaf combinational units, registered FSMs, memory arrays, arithmetic units, and pipeline registers. Top-level integration wrappers that instantiate and connect submodules are excluded from automated generation — VeriGen operates primarily as a code-completion model and showed limited reliability for hierarchical integration tasks, so these top-level modules were implemented independently.

Each generated submodule receives a prompt bounded to ≤120 tokens (mean: 73 tokens), giving the model ≥1900 tokens for generation — enough for any individual component. Every submodule additionally receives a dedicated testbench, while higher-level integration testbenches verify correct interaction among connected submodules.

# 4. Hierarchical Decomposition Workflow

## 4.1 Why Monolithic Generation Fails

Consider the RISC-V Pipeline (M06): its monolithic prompt consumes ≈550 tokens, leaving only 1498 for generation. A correct 5-stage pipelined processor requires 4000–6000 tokens of Verilog, so the model can at best write one-quarter of the needed logic before running out of space — stopping abruptly mid-expression or mid-always-block.

| Module | Prompt (tok) | Output Budget | Req. (tok) | Risk |
| --- | --- | --- | --- | --- |
| N-bit ALU | 80 | 1968 | ~300 | Safe |
| UART TX | 200 | 1848 | ~800 | Safe |
| CRC-32 Engine | 280 | 1768 | ~1200 | Borderline |
| SHA-256 Core | 480 | 1568 | ~3500 | Critical |
| AES-128 Core | 420 | 1628 | ~4200 | Critical |
| Multi-Cycle CPU | 450 | 1598 | ~5000 | Critical |
| 5-Stage RISC-V | 550 | 1498 | ~6000 | Critical |

Table 2. Token budget analysis for representative modules. After decomposition, all submodules stay ≤120 tokens ⇒ ≥1900 output budget.

After hierarchical decomposition, 72 of 150 generated files terminated cleanly with a complete endmodule statement corresponding to the intended module. For the 6B model, several generations additionally appended unrelated continuations (most commonly autogenerated testbenches) after successfully completing the target RTL module; these were conservatively excluded from the strict completion metric, although 14 such files still contained a syntactically complete primary module before continuation drift occurred.

Hierarchical decomposition substantially reduced truncation-related failures and isolated most remaining errors to semantic mistakes in the generated logic. Since submodules are generated and verified independently, compilation and simulation jobs can also execute concurrently across compute nodes, unlike monolithic generation which requires sequential end-to-end compilation and debugging.

Despite this structural improvement, end-to-end functional pass rates remained unexpectedly low: 30% (2B), 0% (6B), and 28% (16B). This disconnect between structural improvement and functional stagnation motivates the error taxonomy in Section 6, and suggests context-window limitations are not the primary barrier to reliable RTL generation — more fundamental semantic and architectural failures persist even after truncation is mitigated.

# 5. Experimental Setup

## 5.1 Models

Three VeriGen checkpoints (2B, 6B, 16B parameters) were obtained from the official repository accompanying the VeriGen paper. VeriGen fine-tunes CodeGen-family autoregressive code-generation models on Verilog corpora. All models use the CodeGen architecture with a 2048-token context window.

Inference is performed using greedy decoding (temperature = 0, n = 1) to ensure deterministic generation.

## 5.2 Infrastructure

All generation jobs were executed on a High Performance Computing (HPC) cluster equipped with NVIDIA H100 80GB SXM5 GPUs. Workloads were managed using the SLURM (Simple Linux Utility for Resource Management) scheduler, which handles distributed job allocation, GPU scheduling, and resource orchestration across compute nodes. The software stack used Python 3.10 with PyTorch (CUDA 12.1) and HuggingFace Transformers/Accelerate.

## 5.3 Verification

Syntax checking uses Icarus Verilog (iverilog) v11. A Python runner iterates all .v files, invokes iverilog with full error capture, and classifies output against the taxonomy in Section 6. Functional testbenches run with vvp, the runtime simulation engine of Icarus Verilog that executes the compiled HDL and verifies circuit behavior against expected outputs; simulations exceeding 10,000 cycles are classified as TIMEOUT.

## 5.4 Prompts

Each of the 50 submodule prompts uses the format established in the original VeriGen paper: a natural-language comment describing function and interface, a complete module header with port declarations, and a partial RTL skeleton.

# 6. Results and Error Taxonomy

## 6.1 Error Taxonomy

Six mutually exclusive outcome classes are defined, ordered by verification stage — increasingly deep levels of correctness: F1 fails at the compiler, F5 compiles and runs but produces wrong answers.

| Code | Stage | Description |
| --- | --- | --- |
| PASS | Simulation | All testbench assertions pass |
| F1 | Compile | Iverilog rejects at compile time |
| F2 | Compile | Missing submodule type (undefined component) |
| F3 | Simulation | Structurally incomplete (missing logic blocks) |
| F4 | Simulation | Non-synthesizable construct (valid simulation, unmanufacturable) |
| F5 | Simulation | Functional mismatch (wrong logic, bad FSM, incorrect operation) |

Table 3. RTL Generation Error Taxonomy (six classes).

Root causes of F1 failures — five iverilog-detectable subtypes were identified across all 150 runs:

- Duplicate declarations (dominant in 2B and 6B): the model enters a degenerate repetition loop, appending _next suffixes to signal names indefinitely, then re-declaring the original, generating cascading compile errors until iverilog “gives up.”

- Wire l-value violations: a wire is incorrectly assigned inside a clocked always block, observed in cpu_regfile (M04, 2B) and rv_if_stage (M06, 16B).

- Undefined macros (exclusive to 6B): the model hallucinates framework-level constants such as `UartParityType, `OR1200_OPERAND_WIDTH, and `DMEM_MSB — symbols from unrelated open-source projects in its training data.

Root causes of F2 failures — unknown module types: the generated RTL is syntactically valid but instantiates submodules absent from the design hierarchy. VeriGen-2B produces three such cases (undefined half_adder/full_adder in alu_addsub, a missing mux2 helper in cpu_alu, an undefined sbox module in aes_shiftrows); VeriGen-16B contributes one case (fft_stage23 delegating to a hallucinated fft_bits module). VeriGen-6B produces no F2 failures because most of its generations fail earlier at F1.

## 6.2 Overall Pass Rates

| Model | F1 | F2 | F3+F4 | F5 | Syntax Pass% | Sim. Pass% |
| --- | --- | --- | --- | --- | --- | --- |
| VeriGen-2B | 25 | 3 | 3 | 4 | 44 | 30 |
| VeriGen-6B | 50 | 0 | 0 | 0 | 0 | 0 |
| VeriGen-16B | 22 | 1 | 4 | 9 | 54 | 28 |
| All | 97 | 4 | 7 | 13 | 32.7 | 19.3 |

Table 4. Model comparison — error counts and pass rates. Syntax Pass% = (50−F1−F2)/50; Simulation Pass% = (50−F1−F2−F3−F4−F5)/50.

VeriGen-6B achieves 0% syntax and functional pass rates, performing substantially worse than both 2B and 16B. This is qualitatively consistent with, though not conclusive evidence for, U-shaped/non-monotonic scaling effects reported in prior inverse-scaling literature; a difference in model architecture makes this reading one of at least two equally plausible explanations. The 6B model frequently generated undefined external macros and unrelated testbench continuations (e.g. `OR1200_OPERAND_WIDTH), possibly reflecting contamination or memorization effects involving macro-heavy Verilog repositories. Ignoring appended testbench continuations, 14/50 generations still contained a complete primary module before corruption or truncation occurred. The original VeriGen paper notes the 6B backbone uses fewer attention heads than the 2B and 16B variants, so a weak-model/architectural explanation is at least as plausible as an inverse-scaling effect; the data here cannot distinguish between the two, and the collapse is presented descriptively rather than as evidence for either account.

16B vs. 2B: the 16B model achieves a much better syntax pass rate (54% vs. 44%) but a slightly lower functional pass rate (28% vs. 30%). Larger capacity reduces F1 compile errors but nearly doubles F5 functional failures (9 vs. 4) — the model generates structurally correct code that produces wrong answers, particularly in FSM designs and cryptographic modules.

## 6.3 Per-Module Results

| ID | Module | 2B | 6B | 16B |
| --- | --- | --- | --- | --- |
| M01 | ALU | 2/4 | 0/4 | 2/4 |
| M02 | Register File | 0/1 | 0/1 | 0/1 |
| M03 | UART TX | 0/3 | 0/3 | 0/3 |
| M04 | Multi-Cycle CPU | 1/6 | 0/6 | 1/6 |
| M05 | Control Unit | 1/2 | 0/2 | 2/2 |
| M06 | RISC-V Pipeline | 2/6 | 0/6 | 2/6 |
| M07 | Cache | 1/3 | 0/3 | 0/3 |
| M08 | Arbiter | 1/2 | 0/2 | 1/2 |
| M09 | AES-128 | 2/9 | 0/9 | 2/9 |
| M10 | SHA-256 | 2/4 | 0/4 | 1/4 |
| M11 | CRC-32 | 1/2 | 0/2 | 1/2 |
| M12 | 8-Pt FFT | 1/4 | 0/4 | 0/4 |
| M13 | Matrix Multiply | 0/2 | 0/2 | 1/2 |
| M14 | Bubble Sort | 1/2 | 0/2 | 1/2 |

Table 5. Per-module functional pass rate (successful generations / total generations).

Zero-pass modules: Register File (M02) and UART TX (M03) fail across all three models in every evaluated generation. UART failures arise from incorrect FSM state-transition sequencing (invalid START→DATA→STOP ordering). Register file failures stem from confusing when reads should be clocked versus unclocked — models write rdata inside clocked always blocks, introducing unintended cycle latency.

Control Unit (M05): VeriGen-16B achieves a perfect 2/2 score, likely because decoder-style control logic and large case structures are commonly observed in Verilog training corpora.

AES-128 (M09) shows the widest error diversity, spanning Galois-field arithmetic, S-box lookup structures, and key-scheduling logic. Models occasionally generate simpler XOR-style arithmetic correctly but consistently fail on lookup-heavy affine transformation components such as aes_sbox_lo.

## 6.4 Code Coverage of Passing Modules

For every module that compiled and simulated successfully, coverage analysis was performed using the Synopsys VCS simulation environment together with Verdi waveform and coverage analysis tools. VCS executed RTL simulations and collected line, condition, toggle, and branch coverage metrics; Verdi was used for detailed post-simulation coverage inspection and debugging.

| Module | Model | Line | Cond | Toggle | Branch |
| --- | --- | --- | --- | --- | --- |
| compare_swap | 2B | 96.9 | 100.0 | 99.2 | 83.3 |
| compare_swap | 16B | 98.7 | 100.0 | 99.2 | 83.3 |
| rv_hazard | 2B | 91.2 | 100.0 | 85.5 | 54.6 |
| rv_hazard | 16B | 99.1 | 100.0 | 98.7 | 83.3 |
| alu_flags | 2B | 98.9 | 87.5 | 98.1 | 50.0 |
| alu_logic | 16B | 97.0 | 100.0 | 86.3 | 83.3 |
| sha256_k_rom | 2B | 91.3 | 50.0 | 99.4 | 88.0 |
| sha256_k_rom | 16B | 95.4 | — | 99.4 | 88.9 |
| crc32_ctrl | 2B | 92.9 | — | 97.7 | 57.7 |
| aes_gf_mul | 2B | 96.7 | — | 99.0 | 50.0 |
| cache_ctrl | 16B | 99.6 | 50.0 | 76.9 | 95.2 |
| fft_twiddle | 2B | 88.3 | — | 87.9 | 56.0 |

Table 6. Representative code coverage (%) for the strongest-performing submodule per category.

Passing submodules achieve consistently high coverage across multiple metrics, increasing confidence that successful simulations correspond to genuinely functional implementations rather than trivial or weakly tested outputs. Lower branch/condition coverage in some FSM-heavy modules suggests common execution paths are well validated while certain edge-case transitions remain underexplored. The “—” entries arise from two causes: in crc32_ctrl and aes_gf_mul, the RTL is straight-line combinational datapath logic without condition-evaluable expressions, so VCS reports no meaningful conditional coverage; in sha256_k_rom (16B) and fft_twiddle, generated case-item width mismatches and improperly sized literals prevented reliable conditional coverage extraction despite successful simulation.

## 6.5 Error Distribution and Token-Failure Correlation

VeriGen-16B reduces the combined F1+F2 failure rate by 17.9% relative to 2B but increases F5 functional failures by 2.25×, suggesting larger models improve structural RTL generation without corresponding gains in semantic correctness. Modules requiring >300 tokens in monolithic form (AES, RISC-V, SHA-256, Multi-Cycle CPU) account for 54.5% of all F1 failures even after decomposition, indicating that hierarchical decomposition reduces truncation but does not fully resolve reasoning difficulties in semantically complex designs.

# 7. Discussion

## 7.1 Decomposition Is Necessary But Not Sufficient

Breaking large designs into bounded submodule prompts reduces truncation and enables structural evaluation, but does not solve semantic generation accuracy. Clear, well-specified leaf-level submodules (e.g. alu_shift, crc32_ctrl) pass reliably in 2B and 16B, while submodules requiring memorized constant tables (AES S-box) or multi-cycle state management (UART FSM) fail consistently across all three models. Notably, alu_shift and crc32_ctrl both pass in 2B and 16B but fail at F1 in 6B, confirming that the 6B collapse is model-specific rather than a reflection of submodule difficulty. VeriGen-2B appears comparatively more stable for localized RTL completion and simpler datapath-style modules, while VeriGen-16B improves structural correctness but continues to exhibit semantic coordination failures in more complex hierarchical systems; the present benchmark size is not sufficient to support definitive deployment recommendations.

## 7.2 Possible Training-Data Contamination Effects in 6B

The 6B model frequently hallucinated macro symbols from unrelated external projects such as OpenRISC (`OR1200_OPERAND_WIDTH), suggesting intermediate model sizes may be especially susceptible to training-data contamination. The 2B model did not exhibit this specific macro-hallucination pattern. A secondary failure mode was observed exclusively in 6B: unsolicited testbench generation. In 14 of the 50 leaf submodule files, the model completed the requested module correctly and then continued generating unrequested _tb modules within the same file, sometimes repeating the testbench two or three times until the token limit was reached mid-generation. This intra-file module pollution caused duplicate module declarations and fused endmodulemodule tokens that iverilog rejects at compile time, directly contributing to 6B's F1 count.

## 7.3 Integration Failure Amplification

Integration tests exhibit systematically worse pass rates than unit tests: of 42 integration-level runs (14 modules × 3 models), none produced a passing result. Any submodule F1 failure propagates to integration failure, creating a compounding effect. Future work should explore hybrid integration strategies in which failed generated submodules are replaced with reference RTL implementations, enabling finer-grained measurement of how individual submodule errors affect the correctness of the assembled system.

## 7.4 Implications for Model Training

- Fine-tuning data should include more diverse, standalone Verilog files rather than macro-heavy framework repositories, so models learn to generate self-contained modules without external symbol dependencies.

- Training-corpus curation should explicitly filter out files containing multi-module definitions, particularly implementation-plus-testbench pairs in a single .v file, to prevent 6B-style intra-file pollution and unsolicited testbench generation.

- Prompt engineering with explicit port-type annotations may reduce wire l-value violations.

## 7.5 Limitations of Completion-Only RTL Models

During integration-level evaluation, VeriGen frequently ignored explicit natural-language instantiation directives inside top-level prompts (e.g. “Instantiate alu_addsub #(N)(...), alu_logic #(N)(...)…”) and instead attempted to regenerate local arithmetic and control logic directly inside the top-level module body. This reflects a limitation of VeriGen's completion-only training objective: unlike instruction-tuned conversational LLMs such as ChatGPT or Claude, VeriGen is trained for next-token RTL completion rather than hierarchical planning, so it can continue local RTL patterns inside incomplete modules but struggles with the global structural reasoning required for top-level system integration. Several top-level integration wrappers were therefore constructed manually to maintain benchmark consistency, while all submodule implementations were generated by the evaluated models.

Taken together, within the completion-only, CodeGen-derived model family evaluated here, neither model scale nor context-window length appears to be the primary limitation in LLM-based RTL generation at realistic hardware complexity. Instead, dominant constraints arise from training-data quality and limited domain-specific semantic reasoning involving FSM sequencing, cryptographic constant handling, and pipeline coordination. This finding is not claimed to generalize to instruction-tuned models, which may reason about hierarchical structure differently.

## 7.6 Scope and Limitations

Findings rest on a single, dated model family (VeriGen 2B/6B/16B, CodeGen-based, completion-only, 2048-token window); modern instruction-tuned models may behave differently and were outside the scope of this study. All results reflect a single greedy-decoding run per submodule (n = 1) — variance across repeated samples was not measured. Extending HierRTLBench to instruction-tuned models (e.g. GPT-4, Claude) with repeated sampling is the primary direction for future work. The taxonomy also does not currently capture lint-level design quality issues such as unintended latch inference or multiply-driven nets, left for future extension.

# 8. Conclusion

HierRTLBench is a hierarchical hardware benchmark suite and evaluation framework for LLM-based Verilog generation, covering 14 diverse modules decomposed into 50 submodules spanning processors, caches, cryptographic cores, DSP accelerators, and communication interfaces. Evaluating VeriGen-2B, 6B, and 16B over 150 leaf submodule verification runs found:

- The 2048-token context limit makes monolithic generation infeasible for 4 of the 14 modules; hierarchical decomposition drastically reduces structural truncation from 100% under monolithic generation to 52% across decomposed outputs (strict metric). Crediting the 14 VeriGen-6B submodules that completed the target module correctly before appending an unsolicited testbench, truncation falls further to 42.6% (lenient metric). All other results in the paper use the strict metric unless noted.

- VeriGen-6B exhibits a complete collapse in functional correctness (0% pass rate), driven not by token-pressure limitations alone but by apparent training-data contamination effects such as undefined macro generation and unsolicited intra-file testbench insertion — aligning with non-monotonic/U-shaped scaling effects reported in prior work, where intermediate-scale models can amplify training-data pathologies instead of mitigating them.

- VeriGen-16B achieves a higher syntax pass rate (54% vs. 44%) but produces roughly twice as many F5 functional failures (9 vs. 4) — a qualitative shift rather than an overall improvement.

- Complex modules involving FSM sequencing (UART, CPU control), cryptographic constant tables (AES, SHA-256), and multi-stage pipeline coordination (RISC-V) resist all three model sizes.

- Passing submodules achieve strong line coverage, but branch and condition coverage remain low, revealing testbench gaps in corner-case and edge-state exercise that leave meaningful portions of the generated logic unvalidated.

The HierRTLBench error taxonomy (F1–F5, PASS) provides a structured vocabulary for diagnosing RTL generation systems, enabling targeted improvements in fine-tuning data curation, prompt engineering, and post-generation repair. The identification of intra-file module pollution as a distinct failure class unique to intermediate-scale models opens a new direction for training-corpus filtering and generation-boundary enforcement in future LLM-based hardware synthesis pipelines.

## Acknowledgments

This work is partially supported by the research project sponsored by Xilinx-AMD. Opinions and conclusions presented are solely those of the authors and are not representative of AMD's roadmap.

# References

[1] S. Thakur, B. Ahmad, H. Pearce, B. Tan, B. Dolan-Gavitt, R. Karri, and S. Garg, “VeriGen: A large language model for Verilog code generation,” ACM Trans. Des. Autom. Electron. Syst., vol. 29, no. 3, Apr. 2024.

[2] J. Tang, J. Qin, K. Thorat, C. Zhu-Tian, Y. Cao, Y. Zhao, and C. Ding, “HiVeGen: Hierarchical LLM-based Verilog generation for scalable chip design,” in Proc. IEEE ICLAD, 2025, pp. 30–36.

[3] M. Liu, N. Pinckney, B. Khailany, and H. Ren, “VerilogEval: Evaluating large language models for Verilog code generation,” in Proc. IEEE/ACM ICCAD, 2023.

[4] S. Liu, W. Fang, Y. Lu, Q. Zhang, H. Zhang, and Z. Xie, “RTLCoder: Outperforming GPT-3.5 in design RTL generation,” in Proc. IEEE LLM Aided Design Workshop (LAD), 2024.

[5] A. Nakkab, S. Q. Zhang, R. Karri, and S. Garg, “Rome was not built in a single step: Hierarchical prompting for LLM-based chip design,” in Proc. ACM/IEEE MLCAD, 2024.

[6] K. Chang, Y. Wang, H. Ren, M. Wang, S. Liang, Y. Han, H. Li, and X. Li, “ChipGPT: How far are we from natural language hardware design,” arXiv:2305.14019, 2023.

[7] Y. Lu, S. Liu, Q. Zhang, and Z. Xie, “RTLLM: An open-source benchmark for design RTL generation with large language models,” arXiv:2308.05345, 2023.

[8] J. Blocklove, S. Garg, R. Karri, and H. Pearce, “Chip-Chat: Challenges and opportunities in conversational hardware design,” in Proc. ACM/IEEE MLCAD, 2023.

[9] Y. Fu, Y. Zhang, Z. Yu, S. Li, Z. Ye, C. Li, C. Wan, and Y. Lin, “GPT4AIGChip: Towards next-generation AI accelerator design automation via LLMs,” in Proc. IEEE/ACM ICCAD, 2023.

[10] E. Nijkamp, B. Pang, H. Hayashi, L. Tu, H. Wang, Y. Zhou, S. Savarese, and C. Xiong, “CodeGen: An Open Large Language Model for Code with Multi-Turn Program Synthesis,” arXiv:2203.13474, 2022.

[11] S. Williams, “The Icarus Verilog compilation system,” github.com/steveicarus/iverilog, 2023.

[14] Y. Tsai, M. Liu, and H. Ren, “RTLFixer: Automatically fixing RTL syntax errors with large language models,” in Proc. 61st ACM/IEEE Design Automation Conference (DAC), 2024.

[15] I. R. McKenzie et al., “Inverse scaling: When bigger isn't better,” Trans. Mach. Learn. Res., 2023, arXiv:2306.09479.

[16] J. Wei, Y. Tay, and Q. V. Le, “Inverse scaling can become U-shaped,” arXiv:2211.02011, 2023.

[17] M. Riddell, A. Ni, and A. Cohan, “Quantifying contamination in evaluating code generation capabilities of language models,” in Proc. 62nd Annual Meeting of the ACL, vol. 1, pp. 14116–14137, Aug. 2024.

[18] T. Han, Z. Wang, C. Fang, S. Zhao, S. Ma, and Z. Chen, “Token-budget-aware LLM reasoning,” in Findings of the ACL 2025, pp. 24842–24855, 2025, arXiv:2412.18547.